APB_UART Core Verification Plan

Prepared by: Hemang Nagpal

Contents

1. APB_UART Verification Plan	2

1.1 APB_UART Core Features	2

2. APB_UART Block Diagram	3

3. Details of UVM Testbench Integration	4

3.1 apb_uart_tb_top	4

3.2 apb_uart_base_test	5

3.3 apb_uart_top_env	5

3.4 apb_uart_virtual_sequencer	5

3.5 apb_uart_reg_block	5

3.6 apb_uart_scoreboard	5

3.7 apb_uart_env	5

3.8 apb_uart_agent	6

3.9 apb_uart_monitor	6

3.10 apb_uart_driver	6

3.11 apb_uart_sequencer	6

3.12 apb_uart_seq_item	6

4. APB_UART Testcases/sequences	6

# 1. APB_UART Verification Plan

## 1.1 APB_UART Core Features

- Compliance with AMBA APB Specification for SoC integration.

- APB slave interface for register access.

- Separate 32×8 Transmit FIFO.

- Separate 32×12 Receive FIFO.

- Programmable FIFO enable/disable mode.

- Integer and Fractional Baud Rate Generator.

- UART transmitter functionality.

- UART receiver functionality.

- Configurable word length (5/6/7/8 bits).

- Configurable parity generation and checking.

- Configurable stop bit generation.

- Break generation support.

- Internal loopback mode.

- TX FIFO interrupt generation.

- RX FIFO interrupt generation.

- Receive timeout interrupt generation.

- Error interrupt generation.

- Interrupt masking and interrupt clearing.

- Error detection:

  - Overrun Error (OE)

  - Framing Error (FE)

  - Parity Error (PE)

  - Break Error (BE)

# 2. APB_UART Block Diagram

Fig1. Block Diagram

# 3. Details of UVM Testbench Integration

## 3.1 apb_uart_tb_top

- Include packages for UVM, APB Master and UART.

- Instantiated interface for APB Master and UART.

- DUT instantiation and its connection with interface.

- Pass interface to the respective VIP.

## 3.2 apb_uart_base_test

- Instantiate top level environment(apb_uart_top_env) and configuration object.

- Update the configuration field as per testcase requirement.

- Print the uvm_topology().

## 3.3 apb_uart_top_env

- Instantiation of APB Master VIP env, APB_UART env, Virtual sequencer, reference model, scoreboard, and RAL Model (apb_uart_adapter, apb_uart_reg_block), predictor.

- Passing the configuration to the component below.

- Connection between RAL Model (apb_uart_reg_block, apb_uart_predictor), coverage model, checkers, and reference model.

## 3.4 apb_uart_virtual_sequencer

- All test case sequences will be executed through the virtual sequencer.

- Installation of the APB Master VIP sequencer, which will be connected to the APB Master VIP driver.

- Installation of the APB_UART sequencer, which will be connected to the APB_UART driver.

## 3.5 apb_uart_reg_block

- It contains the registers, register fields, and offset information for each register in the APB_UART block.

- The mirror () function will be called for data checking purposes and updates will be reflected in both the DUT and the RAL model.

## 3.6 apb_uart_scoreboard

- Retrieve the expected data from the APB Master VIP interface and apb_uart_reference_model.

- Capture the actual data from the apb_uart_monitor.

- Perform the data integrity to compare the expected and actual data value.

## 3.7 apb_uart_env

- The apb_uart_env for the UART includes an instantiation of the apb_uart_agent and configuration object.

- Set the configuration.

## 3.8 apb_uart_agent

- The apb_uart_agent includes the uart_driver, apb_uart_sequencer, apb_uart_monitor and configuration objects.

- The apb_uart_agent is set to active mode.

## 3.9 apb_uart_monitor

- Get the values of all registers and UART interface signals and put them into a sequence item.

- Send the sequence item to the scoreboard for data integrity checking.

- Send the same sequence item to the coverage collector for functional coverage sampling.

## 3.10 apb_uart_driver

- Generate APB_UART transactions based on the sequence item.

- Drive UART transmit and receive stimulus to the DUT.

- Support constrained-random traffic generation for Coverage Driven Verification.

## 3.11 apb_uart_sequencer

- Controls the execution of APB_UART sequences.

- Supports directed and constrained-random stimulus generation.

## 3.12 apb_uart_seq_item

- It includes all the register data fields required for both coverage and expected data generation.

# 4. APB_UART Testcases/sequences

- Refer to the APB_UART_Test_Plan.xlsx for the list of available testcases and sequence details.